DOWNLINK ISAC

Using the actual N310 front panel in your photo, the best connection for your current 4-channel passive downlink ISAC/AoA receiver is to use the four RX2 inputs—one from each RF channel RF0, RF1, RF2, and RF3. The N310 has four independent RX channels.

### Exact physical connection

Notice that the RX2 connector alternates left/right on the front panel:

| Array element | N310 channel | Physical connector in your photo | UHD RX channel |
| --- | --- | --- | --- |
| Top Ant 0 | RF0 | RX2 — right connector | 0 |
| Top Ant 1 | RF1 | RX2 — left connector | 1 |
| Top Ant 2 | RF2 | RX2 — right connector | 2 |
| Top Ant 3 | RF3 | RX2 — left connector | 3 |

For current UHD releases, Ettus maps RF0–RF3 to the four receiver channels; on UHD 3.13+ the subdevice mapping is RF0=A:0, RF1=A:1, RF2=B:0, RF3=B:1.

So looking directly at your N310:

USRP N310 FRONT PANEL

RF0                  RF1                  RF2                  RF3

┌────────────┐       ┌────────────┐       ┌────────────┐       ┌────────────┐

│ TX/RX  RX2 │       │ RX2  TX/RX │       │ TX/RX  RX2 │       │ RX2  TX/RX │

│   ○     ●  │       │  ●     ○   │       │   ○     ●  │       │  ●     ○   │

└────────────┘       └────────────┘       └────────────┘       └────────────┘

↑             ↑                           ↑             ↑

│             │                           │             │

Ant 0         Ant 1                       Ant 2         Ant 3

Here ● is the connector I recommend using.

Therefore:

TOP ROW OF YOUR ANTENNA ARRAY

Ant 0          Ant 1          Ant 2          Ant 3

│              │              │              │

│ coax         │ coax         │ coax         │ coax

│              │              │              │

▼              ▼              ▼              ▼

RF0/RX2        RF1/RX2        RF2/RX2        RF3/RX2

N310 Ch 0      N310 Ch 1      N310 Ch 2      N310 Ch 3

### Very important: don't use the upper LO connectors

The four large gold connectors at the top marked:

LO IN 0/1

TX

RX

LO IN 2/3

TX

RX

are external local-oscillator inputs. They are not antenna inputs. For your present experiment, leave all four LO-IN connectors alone. Ettus documents those connectors as external LO inputs used for applications such as improving inter-channel phase alignment.

Likewise, for your receive-only passive ISAC experiment, you do not need to connect the TX/RX jacks.

### Why RX2 instead of TX/RX?

Both TX/RX and RX2 can be used as receive inputs, but RX2 is the dedicated receive path. For a passive receiver that is not transmitting, it is the cleaner choice. Ettus publishes separate receive-noise-figure specifications for the TX/RX and RX2 paths, confirming that both are valid receive inputs.

Your software should therefore select: RX2, RX2, RX2, RX2 : for channels 0–3.

With the package we prepared, that corresponds to:

python3 ocudu_array_monitor.py \

--device-args "type=n3xx,mgmt_addr=192.168.30.2,addr=192.168.10.2,second_addr=192.168.20.2" \

--channels 0,1,2,3 \

--antennas RX2,RX2,RX2,RX2 \

--element-labels Top0,Top1,Top2,Top3

### Keep the physical antenna order exactly the same as channel order

For AoA, this matters a great deal.

Connect:

leftmost selected antenna  → RF0

next antenna               → RF1

next antenna               → RF2

rightmost selected antenna → RF3

Do not accidentally do something such as:

Ant 0 → RF0

Ant 1 → RF2

Ant 2 → RF1

Ant 3 → RF3

The waterfall would still work, but the spatial phase progression would be wrong and the AoA estimate could become inaccurate or even reverse direction.

For your array, I recommend:

TOP ROW

Ant 0 ─── Ant 1 ─── Ant 2 ─── Ant 3

│         │         │         │

RF0       RF1       RF2       RF3

because the current AoA implementation is based on a one-dimensional linear array.

### Cables

Use four 50-ohm coax cables with the same electrical length:

Array Ant 0 ── equal cable ── RF0/RX2

Array Ant 1 ── equal cable ── RF1/RX2

Array Ant 2 ── equal cable ── RF2/RX2

Array Ant 3 ── equal cable ── RF3/RX2

For AoA, cable length and phase consistency are much more important than they would be for ordinary independent receivers.

Ideally: Cable 0 = Cable 1 = Cable 2 = Cable 3

same manufacturer

same coax type

same nominal length

same connector/adaptor arrangement

If one channel has an extra adapter or noticeably different cable length, that creates an additional phase offset that the AoA estimator may interpret as spatial phase.

### Your complete setup should therefore look like this

2 × 8 ANTENNA ARRAY

TOP:

[A0]────[A1]────[A2]────[A3]────[A4]────[A5]────[A6]────[A7]

│       │       │       │

│       │       │       │

│       │       │       └──────────────────────────────┐

│       │       └───────────────────────┐              │

│       └─────────────────┐             │              │

└──────────┐              │             │              │

│              │             │              │

▼              ▼             ▼              ▼

RF0 / RX2      RF1 / RX2      RF2 / RX2      RF3 / RX2

UHD Ch 0       UHD Ch 1       UHD Ch 2       UHD Ch 3

┌───────────────────────────────────────────────────────┐

│                    USRP N310                          │

└───────────────────────────────────────────────────────┘

│

│ 10 GbE

▼

HOST PC

│

▼

OCUDU Passive ISAC GUI

Antennas 4–7 on the top row and all eight bottom-row antennas remain disconnected for this experiment.

### One hardware safety point

Your N310 front panel itself says:

RX INPUT MAX -15 dBm

and Ettus lists approximately −15 dBm as the maximum recommended receiver input at the 1-dB compression point.

Because you are doing passive downlink sensing near an operating 5G gNB, don't place this receive array extremely close to a high-power transmitting antenna without checking received power first.

### For your AoA experiment specifically

There is one more important N310 characteristic: Ettus notes that the N310 does not inherently guarantee the same RF phase alignment between channels across separate initialization runs; inter-channel phase can change when the radio is reinitialized. An external LO can reduce that ambiguity.

Therefore, with your present setup, use this procedure:

Connect all four antennas

↓

Start N310

↓

Start one 4-channel UHD session

↓

Put a known source at array broadside / 0°

↓

Measure phase offsets Ch1-Ch0, Ch2-Ch0, Ch3-Ch0

↓

Use these as calibration offsets

↓

Perform AoA / sensing experiments

Do not restart the UHD session between calibration and measurement if possible.

For the hardware shown in your photo, the four connectors you want are therefore unequivocally:

RF0 RX2 → RF1 RX2 → RF2 RX2 → RF3 RX2, connected in the same left-to-right order as four adjacent elements of your top antenna row.